Question 1

10

Considering the architecture of a RISC-V processor RV32G with single-precision FP operations presented in the following:

• Integer ALU: 1 clock cycle

• Data memory: 1 clock cycle

• FP multiplier unit: pipelined 5 stages

• FP arithmetic unit: pipelined 3 stages

• FP divider unit: not pipelined unit that requires 5 clock cycles

• branch delay slot: 1 clock cycle, and the branch delay slot disabled

• forwarding enabled

• it is possible to complete instruction EXE stage in an out-of-order fashion.

Using the following code fragment, show the timing of the presented loop-based program and compute how many cycles does this program take to execute?

for (i = 0; i < 100; i++) {

v4[i] = v1[i]/v2[i];

v5[i] = ((v4[i]+v3[i])/v[2]); }

|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Clock cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V1: .word “100 values” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| V2: .word “100 values” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| V3: .word “100 values” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| V4: .word “100 zeroes” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| V5: .word “100 zeroes” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| .text |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| main: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| addi x6, x0, 100 | F | D | E | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| la x1, V1 |  | F | D | E | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 |
| la x2, V2 |  |  | F | D | E | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |
| la x3, V3 |  |  |  | F | D | E | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |
| la x4, V4 |  |  |  |  | F | D | E | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |
| la x5, V5 | D | E | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |
| loop: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| flw f1, 0(x1) | F | D | E | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
| flw f2, 0(x2) |  | F | D | E | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
| fdiv f4, f1, f2 |  |  | F | D | s | d | d | d | d | d | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6 |
| fsw f4, 0(x4) |  |  |  | F | s | D | E | s | s | s | s | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
| flw f3, 0(x3) |  |  |  |  |  | F | D | s | s | s | s | E | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
| fadd f5, f4, f3 |  |  |  |  |  |  | F | s | s | s | s | D | s | A | A | A | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 |
| fdiv f5, f5, f2 |  |  |  |  |  |  |  |  |  |  |  | F | s | D | s | s | d | d | d | d | d | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 |
| fsw f5, 0(x5) |  |  |  |  |  |  |  |  |  |  |  |  |  | F | s | s | D | E | s | s | s | s | M | W |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
| addi x1, x1, 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | F | D | s | s | s | s | E | M | W |  |  |  |  |  |  |  |  |  |  |  | 1 |
| addi x2, x2, 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | F | s | s | s | s | D | E | M | W |  |  |  |  |  |  |  |  |  |  | 1 |
| addi x3, x3, 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | F | D | E | M | W |  |  |  |  |  |  |  |  |  | 1 |
| addi x4, x4, 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | F | D | E | M | W |  |  |  |  |  |  |  |  | 1 |
| addi x5, x5, 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | F | D | E | M | W |  |  |  |  |  |  |  | 1 |
| addi x6, x6, -1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | F | D | E | M | W |  |  |  |  |  |  | 1 |
| bnez x6, loop |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | F | s | D | E | M | W |  |  |  |  | 2 + 1 |
| nop |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | F | - | - | - | - |  |  |  |  |
| TOTAL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2915 |

5

Question 2

Considering the couple of instructions:

flw  f2, 0(x2)

fdiv f4, f1, f2

what type of hazard is caused by the use of register f2, and how is it resolved? Justify your answer.

C’è una dipendenza di dato per f2, che causa un hazard RAW. Sto cercando di leggere f2 dopo che viene scritto da un’istruzione precedente. Ciò accade parecchie volte per altri registri (f1).

Per risolvere questo problema, il processore introduce uno stallo, dovuto alla dipendenza di dato.

Cosa succede tra la div e la store di f5? Che tipologia di hazard si presenta.

Idem come sopra, per f5.

Siccome il dato viene calcolato da un’unità precedente, si ha un data hazard (RAW, sto leggendo il dato che viene calcolato dalla divisione). Crea una prima fase dovuta agli stalli di dato e uno stallo dovuto all’accesso contemporaneo in memoria (strutturale). Si risolve introducendo stalli per data e structural hazard (stessa unità di memoria utilizzata in contemporanea).

10

Question 3

Considering the previous program and the superscalar processor architecture described in the following, complete the first two iterations of the program:

Processor architecture:

  - Issue 2 instructions per clock cycle

  - jump instructions require 1 issue

  - handle 2 instructions commit per clock cycle

  - timing facts for the following separate functional units:

    - 1 Memory address  1 clock cycle

    - 1 Integer ALU 1 clock cycle

    - 1 Jump unit 1 clock cycle

    - 1 FP multiplier unit, which is pipelined: 5 stages

    - 1 FP divider unit, which is not pipelined: 5 clock cycles

    - 1 FP Arithmetic unit, which is pipelined: 3 stages

  - Branch  prediction is always correct

  - There are no cache misses

  - There are 2 CDB (Common Data Bus).

| # iteration |  | Issue | EXE | MEM | CDB x2 | COMMIT x2 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | flw f1, 0(x1) |  |  |  |  |  |
| 1 | flw f2, 0(x2) |  |  |  |  |  |
| 1 | fdiv f4, f1, f2 |  |  |  |  |  |
| 1 | fsw f4, 0(x4) |  |  |  |  |  |
| 1 | flw f3, 0(x3) |  |  |  |  |  |
| 1 | fadd f5, f4, f3 |  |  |  |  |  |
| 1 | fdiv f5, f5, f2 |  |  |  |  |  |
| 1 | fsw f5, 0(x5) |  |  |  |  |  |
| 1 | addi x1, x1, 4 |  |  |  |  |  |
| 1 | addi x2, x2, 4 |  |  |  |  |  |
| 1 | addi x3, x3, 4 |  |  |  |  |  |
| 1 | addi x4, x4, 4 |  |  |  |  |  |
| 1 | addi x5, x5, 4 |  |  |  |  |  |
| 1 | addi x6, x6, -1 |  |  |  |  |  |
| 1 | bnez x6, loop |  |  |  |  |  |
| 2 | flw f1, 0(x1) |  |  |  |  |  |
| 2 | flw f2, 0(x2) |  |  |  |  |  |
| 2 | fdiv f4, f1, f2 |  |  |  |  |  |
| 2 | fsw f4, 0(x4) |  |  |  |  |  |
| 2 | flw f3, 0(x3) |  |  |  |  |  |
| 2 | fadd f5, f4, f3 |  |  |  |  |  |
| 2 | fdiv f5, f5, f2 |  |  |  |  |  |
| 2 | fsw f5, 0(x5) |  |  |  |  |  |
| 2 | addi x1, x1, 4 |  |  |  |  |  |
| 2 | addi x2, x2, 4 |  |  |  |  |  |
| 2 | addi x3, x3, 4 |  |  |  |  |  |
| 2 | addi x4, x4, 4 |  |  |  |  |  |
| 2 | addi x5, x5, 4 |  |  |  |  |  |
| 2 | addi x6, x6, -1 |  |  |  |  |  |
| 2 | bnez x6, loop |  |  |  |  |  |

5

Question 4

Considering the code segment presented in the previous table, if we assume that the Reorder Buffer (ROB) has a size of 16 entries and is empty at the start of the code in the table, what is the first instruction that should stall during the program’s execution? Justify your answer.