Question 1

Considering the architecture of a RISC-V processor RV32G with single-precision FP operations presented in the following:

• Integer ALU: 1 clock cycle

• Data memory: 1 clock cycle

• FP multiplier unit: pipelined 8 stages

• FP arithmetic unit: pipelined 4 stages

• FP divider unit: not pipelined unit that requires 8 clock cycles

• branch delay slot: 1 clock cycle, and the branch delay slot disabled

• forwarding enabled

• it is possible to complete instruction EXE stage in an out-of-order fashion.

Using the following code fragment, show the timing of the presented loop-based program and compute how many cycles does this program take to execute?

for (i = 0; i < 100; i++) {

v4[i] = v1[i] + v2[i];

v5[i] = v2[i] * v3[i];

v6[i] = v1[i] / v3[i]; }

|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Clock cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V1: .word “100 values” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| V2: .word “100 values” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| V3: .word “100 values” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| V4: .word “100 zeroes” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| .text |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| main: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| addi x7, x0, 100 | F | D | M | E | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 |
| la x1, V1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |
| la x2, V2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |
| la x3, V3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |
| la x4, V4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |
| la x5, V5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |
| la x6, V6 |  |  |  | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |
| loop: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| flw f1, 0(x1) | F | D | E | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
| flw f2, 0(x2) |  | F | D | E | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
| fadd.s f4, f1, f2 |  |  | F | D | S | A | A | A | A | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 |
| flw f3, 0(x3) |  |  |  | F | S | D | E | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| fsw f4, 0(x4) |  |  |  |  |  | F | D | E | S | S | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
| fmul.s f5, f2, f3 |  |  |  |  |  |  | F | D | X | X | X | X | X | X | X | X | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6 |
| fsw f5, 0(x5) |  |  |  |  |  |  |  | F | D | S | E | S | S | S | S | S | S | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
| fdiv.s f6, f1, f3 |  |  |  |  |  |  |  |  | F | S | D | / | / | / | / | / | / | / | / | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |
| fsw f6, 0(x6) |  |  |  |  |  |  |  |  |  |  | F | D | E | S | S | S | S | S | S | S | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
| addi x1, x1, 4 |  |  |  |  |  |  |  |  |  |  |  | F | D | S | S | S | S | S | S | S | E | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
| addi x2, x2, 4 |  |  |  |  |  |  |  |  |  |  |  |  | F | S | S | S | S | S | S | S | D | E | M | W |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
| addi x3, x3, 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | F | D | E | M | W |  |  |  |  |  |  |  |  |  |  |  | 1 |
| addi x4, x4, 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | F | D | E | M | W |  |  |  |  |  |  |  |  |  |  | 1 |
| addi x5, x5, 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | F | D | E | M | W |  |  |  |  |  |  |  |  |  | 1 |
| addi x6, x6, 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | F | D | E | M | W |  |  |  |  |  |  |  |  | 1 |
| addi x7, x7, -1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | F | D | E | M | W |  |  |  |  |  |  |  | 1 |
| bnez x7, loop |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | F | S | D | E | M | W |  |  |  |  |  | 2 + 1 |
| nop |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | F | - | - | - | - |  |  |  |  | 1 |
| TOTAL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2817 |

Question 2

Considering the previous program, and in particular the following two instructions:

addi  x7, x7, -1

bnez  x7, loop

what type of hazard is caused by the use of register x7, and how is it resolved? Justify your answer.

Question 3

Considering the previous program, and in particular the following two instructions:

fdiv.s f6, f1, f3

fsw  f6, 0(x6)

what type of hazard is caused by the use of register x7, and how is it resolved? Justify your answer.